## Hydro Morph-Continuum Studio: Multi-Scale Sediment & Rheology Analytics

Phase 1:  Exner- Morpho dynamics

What pushes me to delve in this computational realm ?
My drive to build the HydroMorph suite stems from a fundamental frustration with fragmented, black-box engineering software. Sediment transport is not a single-scale phenomenon; it is a non-linear, violently dynamic system where micro-scale turbulent bursts and bed shear stresses dictate macro-scale riverbed scouring. I wanted to stop looking at these processes in empirical isolation and architect a unified, deterministic morphodynamic engine from the ground up, mathematically coupling hydrodynamics and morphodynamics into a continuous feedback loop.

This suite is the natural culmination of my computational journey. Building Boundary-Layer Studio allowed me to master the micro-physics of viscous sublayers and Reynolds stresses, while Delta Pulse provided the framework to handle meso-scale shallow-water hydroinformatics. I realized the next logical frontier was mathematically bridging these extremes—translating the benthic shear forces calculated at the boundary layer into actual bed sediment fluxes to create a living, breathing continuum model of channel evolution.

The civil engineering industry relies heavily on legacy software that abstracts physical reality behind outdated empirical coefficients and steady-state approximations. I wanted to tear down that abstraction. My goal was to engineer a system driven entirely by first principles, directly solving the hyperbolic partial differential equations of the coupled 1D Saint-Venant and Exner equations. By integrating the Shields criterion for critical entrainment with dynamic sediment flux gradients (∂qs/∂x), I am letting raw continuum mechanics and differential calculus dictate the spatiotemporal shifting of the riverbed.

Most importantly, I wanted to prove that commercial-grade morphodynamics does not require a massive server cluster or an expensive workstation. By aggressively exploiting NumPy’s C-based vectorization, minimizing Python-level loops, and ruthlessly optimizing my finite-difference matrix operations, I reduced the algorithmic overhead exponentially. I explicitly designed this high-density numerical solver to execute shock-capturing schemes and simulate spatiotemporal bed-evolution in real-time on a basic, commercially available Intel Core i3 laptop without thermal throttling.

To achieve this hardware efficiency, I had to completely rethink traditional synchronous application architecture. Instead of choking the UI thread with heavy transient loops, I decoupled the core physics engine from the frontend. By isolating the dense BLAS/LAPACK tensor operations and time-stepping integrations into asynchronous background processes, the shock-capturing algorithms run at maximum CPU-cycle efficiency. This strict decoupling ensures the Streamlit interface remains completely fluid and responsive, dynamically rendering the bed profiles even on low-tier silicon.

The true engineering thrill of this project lies in taming the severe numerical instability inherent in morphodynamic coupling. Hydrodynamic waves propagate magnitudes faster than a riverbed deforms, making the simultaneous integration of these systems a nightmare of Courant–Friedrichs–Lewy (CFL) condition violations and numerical dispersion. Solving this through morphological acceleration factors (MorFac), dynamic adaptive time-stepping (dt), and Total Variation Diminishing (TVD) limiters wasn't just a coding task; it was a deep dive into Riemann solvers and rigorous algorithmic stability management.

Ultimately, building this suite is my way of proving that the future of civil engineering lies in algorithmic rigor and computational self-reliance. I am not merely writing scripts; I am architecting a highly optimized, multi-scale, tensor-ready physics engine. This project is my definitive statement that a single developer armed with an i3 laptop, vectorization logic, and a relentless obsession with continuum mechanics can build deterministic, high-performance tools that fundamentally disrupt and rival established industry standards.

- Architectural Overview: Hydrodynamic State Engine (Phase 1.1)

The Hydrodynamic State Engine is the computational bedrock of the morphodynamic suite. Its sole responsibility is to deterministically solve the flow field over a highly dynamic, ever-changing topography. Because the bed geometry (zb) deforms at every morphological time step, the hydrodynamic solver cannot rely on steady-state assumptions; it must transiently adapt to changing cross-sections, slopes, and friction parameters without failing due to numerical dispersion.

Here is the rigorous breakdown of its mathematical and algorithmic architecture.

The Governing Physics: 1D Saint-Venant Equations

The engine operates by solving the fully dynamic, 1D Shallow Water Equations (Saint-Venant equations), which are derived from the depth-averaged Navier-Stokes equations.

To maximize computational efficiency and allow for C-level NumPy vectorization, the system casts these hyperbolic partial differential equations (PDEs) into a conservative vector form:

∂U∂t+∂FU∂x=SU

Where the state vector U, flux vector F, and source term vector S are defined as:

U=AQ, F=QQ2A+gI1, S=0gAS0-Sf+gI2

Continuity (Mass Conservation): Ensures the exact balancing of water entering and leaving a control volume (A is wetted area, Q is discharge).

Momentum Conservation: Balances the inertial forces (∂Q/∂t), convective acceleration (∂Q2/A/∂x), hydrostatic pressure gradients (gI1), bed slope (S0), and friction slope (Sf).

Dynamic Variables: Flow depth (h), cross-sectional average velocity (U=Q/A).

Algorithmic Mechanism & Discretization

To solve these non-linear PDEs on an i3 processor without thermal throttling, the engine avoids explicit Python for loops over the spatial domain. Instead, the channel is discretized into a 1D grid of N nodes (Δx).

Spatial Discretization & Shock-Capturing: The solver utilizes a Finite Volume Method (FVM) or a robust finite-difference scheme like MacCormack. This allows the engine to handle flow discontinuities—such as hydraulic jumps or transitions between subcritical (Fr<1) and supercritical (Fr>1) flow—without crashing.

Friction & Energy Dissipation (Sf): The flow field's energy loss is dictated by bed roughness. The engine calculates the friction slope using the Darcy-Weisbach equation (for rigorous boundary layer coupling) or the Manning's equation, iteratively updating the hydraulic radius (R) as the bed aggrades or degrades.

Boundary Value Enforcement:

Upstream (Neumann/Dirichlet): Inflow discharge hydrograph Q0,t.

Downstream (Dirichlet): Stage-discharge rating curve or a fixed tidal boundary hL,t.

Numerical Stability (The CFL Constraint)

The core engineering challenge of this module is preventing matrix explosions when the flow becomes highly transient. The explicit time-stepping algorithm (Δt) is dynamically governed by the Courant–Friedrichs–Lewy (CFL) condition.

Δt=CrΔxmax|U|+gh

The engine continuously monitors the maximum celerity (velocity + wave speed) across all arrays. If the flow accelerates (e.g., due to scouring creating a steep drop), the solver automatically throttles down Δt to ensure the Courant number (Cr) remains strictly ≤1, guaranteeing algorithmic stability.

Output Handoff (The Coupling Variables)

At the end of each hydrodynamic time step, the engine outputs fully synchronized NumPy arrays of the flow state. These arrays represent the driving forces that will be injected directly into the Shear & Transport Capacity Engine:

Flow Depth Array (h): Water surface elevation minus bed elevation.

Velocity Array (U): Cross-sectional flow velocities.

Hydraulic Radius Array (R): Used for calculating the exact shear stress distribution along the wetted perimeter.

Energy Gradient/Friction Slope Array (Sf): The critical parameter required to compute the basal bed shear stress (τb=γRSf), which ultimately dictates particle entrainment in the next sub-model.

FUNDAMENTAL DEFINITIONS AND GOVERNING FORMULAE OF THE SHEAR AND TRANSPORT CAPACITY ENGINE ( Phase 1.2)

Definition: The Shear and Transport Capacity Engine (Flux Module) constitutes the constitutive interface between hydrodynamics and morphodynamics. It quantifies the basal tangential forces exerted by a fluid continuum on a granular boundary and determines the resulting volumetric flux of solid particles when these forces exceed the threshold of motion.

Fundamental terminology:

Bed shear stress: The tangential frictional force per unit area exerted by the flowing fluid on the channel bed.

Critical shear stress: The threshold boundary shear stress at which hydrodynamic forces (drag and lift) overcome the stabilizing gravitational and frictional forces on a sediment particle, initiating motion.

Shields parameter: The dimensionless ratio of fluid mobilizing forces to particle stabilizing forces.

Sediment transport capacity: The maximum volumetric rate of sediment transport per unit channel width that a given flow state can sustain under equilibrium conditions.

Bedload: The fraction of the total sediment load that moves by rolling, sliding, or saltating in continuous or intermittent contact with the bed.

Governing formulae:

Basal Bed Shear Stress (1D Formulation):

τb=ρgRSf

τb=ρgRSf

Where: τb= bed shear stress (N/m2 or Pa) ρ= density of water (kg/m3) g= acceleration due to gravity (m/s2) R= hydraulic radius (m) Sf= friction slope or energy gradient (dimensionless)

Dimensionless Bed Shear Stress (Shields Parameter):

τ*=τbρs-ρgd50

τ*=τbρs-ρgd50

Where: τ*= dimensionless bed shear stress (dimensionless) ρs= density of sediment particles (kg/m3) d50= median particle diameter (m)

Critical Shields Parameter (Incipient Motion):

τc*=fRe*

τc*=fRe*

Where: τc*= critical dimensionless shear stress (dimensionless) Re*=u*d50ν= boundary Reynolds number (dimensionless) u*=τbρ= shear velocity (m/s) ν= kinematic viscosity of water (m2/s)

Meyer-Peter and Müller (MPM) Bedload Transport Capacity:

qs*=8τ*-τc*3/2

qs*=8τ*-τc*3/2

Where: qs*=qsρsρ-1gd503= dimensionless Einstein bedload number (dimensionless) qs= volumetric sediment transport rate per unit width (m2/s)

Assumptions and validity:

The formulation assumes the fluid is incompressible and Newtonian.

The MPM equation is strictly valid for steady, uniform or gradually varied flow over non-cohesive sediment (sand and gravel) where transport occurs primarily as bedload.

It assumes a single representative grain size (d50), neglecting hiding and exposure effects present in poorly sorted (widely graded) mixtures.

The energy slope Sfaccurately represents the local rate of energy dissipation due to skin friction, ignoring form drag unless explicitly partitioned.

Boundary of applicability: The MPM relation fails when suspended load becomes dominant (typically when u*>ws, where wsis the particle fall velocity). The 1D formulation is insufficient in highly turbulent, 3D separating flows where secondary currents dictate bed shear distributions.

Mathematical classification: The basal shear stress equation is an algebraic balance of macroscopic forces. The Shields parameter is a dimensionless algebraic group. The MPM equation is an empirical, non-linear constitutive algebraic relation fitted to experimental data.

Derivation 1: Basal Bed Shear Stress (τb)

Starting principle: Conservation of linear momentum for a fluid control volume in steady, uniform flow.

Assumptions: Flow is fully developed, incompressible, and steady (∂/∂t=0, ∂/∂x=0).

Coordinate system: x-axis aligned with the channel bed (slope angle θ), z-axis normal to the bed.

Consider a fluid control volume of length Δx, cross-sectional area A, and wetted perimeter P.

The forces acting in the x-direction are the weight component of the fluid (Fg) and the resisting boundary shear force (Fτ). Hydrostatic pressure forces cancel out in uniform flow.

Gravitational force component:

Fg=Wsinθ=ρgAΔxsinθ

Fg=Wsinθ=ρgAΔxsinθ

Resisting shear force at the boundary:

Fτ=τbPΔx

Fτ=τbPΔx

Applying equilibrium (Fx=0):

ρgAΔxsinθ-τbPΔx=0

ρgAΔxsinθ-τbPΔx=0

Simplifying and solving for τb:

τb=ρgAPsinθ

τb=ρgAPsinθ

Substituting the definition of hydraulic radius R=A/Pand approximating sinθ≈tanθ=S0(bed slope) for small angles. In gradually varied flow, the bed slope S0is generalized to the energy friction slope Sf:

τb=ρgRSf

τb=ρgRSf

Derivation 2: The Shields Parameter (τ*)

Starting principle: Static equilibrium of a solitary spherical particle resting on the bed.

The fluid exerts a drag force (FD) proportional to the projected area and dynamic pressure, and a lift force (FL).

FD∝ρu2d2

FD∝ρu2d2

The near-bed velocity uis proportional to the shear velocity u*=τb/ρ, therefore FD∝τbd2.

The stabilizing force is the submerged weight of the particle (FW):

FW=ρs-ρgπd36∝ρs-ρgd3

FW=ρs-ρgπd36∝ρs-ρgd3

Incipient motion occurs when the moment of the destabilizing forces equals the moment of the stabilizing forces about a pivot point. The ratio of the driving force to the stabilizing force yields the dimensionless Shields parameter:

τ*=FDFW∝τbd2ρs-ρgd3=τbρs-ρgd

τ*=FDFW∝τbd2ρs-ρgd3=τbρs-ρgd

B. PHYSICAL INTERPRETATION The basal shear stress τbrepresents the extraction of momentum from the fluid flow by the boundary roughness. Physically, the term ρgRSfindicates that shear stress increases with the depth of the flow (represented by R) and the rate of energy loss (Sf).

The Shields parameter τ*physically represents the competition between hydrodynamics and gravity. When τ*>τc*, the fluid energy overcomes the mass and interlocking friction of the particle, forcing it into motion.

The MPM equation dictates that sediment flux qsdoes not scale linearly with flow velocity. The 3/2exponent demonstrates a highly non-linear response; a marginal increase in shear stress beyond the critical threshold results in a massive exponential increase in sediment transport.

MATHEMATICAL REPRESENTATION

The scalar bed shear stress τbutilized in 1D morphodynamics is a spatial reduction of the second-order Cauchy stress tensor σ. Let xibe Cartesian coordinates where x1aligns with the primary flow direction and x3is normal to the bed boundary.

The Cauchy stress tensor is defined as:

σij=-pδij+τij

σij=-pδij+τij

Where: σij= total stress tensor p= thermodynamic pressure (scalar) δij= Kronecker delta (identity tensor) τij= deviatoric viscous stress tensor

For an incompressible Newtonian fluid, the constitutive equation for the deviatoric stress is:

τij=μ∂ui∂xj+∂uj∂xi

τij=μ∂ui∂xj+∂uj∂xi

Where μis dynamic viscosity and uiis the velocity vector field.

MECHANICAL FORMULATION In turbulent open-channel flow, the velocity field is decomposed using Reynolds averaging into a mean component uiand a fluctuating component ui'. The total effective shear stress tensor τijtotalincorporates both viscous momentum transfer and turbulent momentum transfer (Reynolds stresses):

τijtotal=μ∂ui∂xj+∂uj∂xi-ρui'uj'

τijtotal=μ∂ui∂xj+∂uj∂xi-ρui'uj'

Where -ρui'uj'is the Reynolds stress tensor.

To determine the scalar boundary shear stress τb, we define the boundary surface with an outward-pointing unit normal vector nj. The traction vector tiacting on this surface is:

ti=σjinj

ti=σjinj

At the rigid impermeable boundary (z=0), the normal vector is nj=0,0,-1. The tangential (shear) component of the traction vector aligns with the flow direction x1.

DERIVATION AND VERIFICATION Evaluating the stress tensor exactly at the boundary (z=x3=0), the velocity fluctuations vanish due to the no-slip condition (ui'=0). The Reynolds stresses drop to zero, leaving only the viscous stresses. Assuming the mean flow is unidirectional (u1x3) and varies only in the normal direction:

τ31total=μ∂u1∂x3

τ31total=μ∂u1∂x3

Therefore, the macroscopic scalar τbused in the 1D Exner engine is mathematically equivalent to the 13-component of the total stress tensor evaluated at the wall:

τb=τ31totalz=0=μ∂u1∂zz=0

τb=τ31total​z=0=μ∂u1∂z​z=0

This verifies that the 1D empirical formulation (τb=ρgRSf) is a spatially integrated surrogate for the local tensorial momentum flux exactly at the continuum boundary.

The system models two distinct continuous phases interacting at a dynamic boundary: the fluid continuum (water) governed by the Navier-Stokes/Saint-Venant equations, and the granular continuum (riverbed) governed by the Exner equation.

The spatial domain consists of the fluid depth hx,tand the bed elevation zbx,t. The Shear and Transport Capacity Engine acts as the local constitutive closure that couples the kinematics of the fluid to the deformation of the bed.

B. LOCAL GOVERNING EQUATIONS The fundamental governing equation for the morphological deformation is the conservation of sediment mass, formulated locally as the Exner Equation.

Consider a control volume of the riverbed of length dx, width B, and active layer thickness η. Sediment enters with volumetric flux qsinand leaves with qsout. The conservation of sediment mass dictates:

∂Vs∂t=qsin-qsout

∂Vs∂t=qsin-qsout

Expressing the solid volume per unit width as 1-λzbdx, where λis the bed porosity, and taking the limit as dx→0:

1-λ∂zb∂t=-∂qs∂x

1-λ∂zb∂t=-∂qs∂x

Rearranging into the standard differential form:

∂zb∂t+11-λ∇⋅qs=0

∂zb∂t+11-λ∇⋅qs=0

CLOSURE AND PHYSICAL CONSISTENCY :The Exner equation contains two unknowns: the bed elevation zband the sediment flux vector qs. The system is under-determined and physically open.

The Flux Module provides the required constitutive relation to close the mathematical system. By linking the fluid momentum (via τb) to the sediment flux (via the MPM equation qs=fτb), the system becomes solvable.

The coupled closed system is defined sequentially as:

Hydrodynamic field equations solve for Uand h.

The energy gradient computes τbvia τb=ρgRSf.

The constitutive flux module evaluates qs=fτb.

The divergence of the flux field ∇⋅qsis inserted into the Exner equation.

The Exner equation is integrated forward in time to update the spatial geometry zb.

CONNECTION TO ENGINEERING APPLICATIONS In the specific application of the HydroMorph suite, this continuum formulation allows for the shock-capturing simulation of transient riverbed morphodynamics. By representing sediment transport not as discrete moving particles, but as a continuous Eulerian flux field qs, the engine can utilize high-performance finite-difference methods (like MacCormack or TVD limiters) to compute the spatial derivative ∂qs/∂x.

This formulation fundamentally dictates that localized acceleration of fluid over a bed perturbation increases local τb, increasing local qs. If ∂qs/∂x>0(flux accelerates), the Exner equation mandates that ∂zb/∂t<0, modeling the physical scouring of the riverbed geometry.

SECTION 1 — FUNDAMENTAL DEFINITIONS AND GOVERNING FORMULAE OF THE Exner Bed-Evolution Engine and Morphological Stability Manager ( Phase 1.3)

Definition: The Exner Bed-Evolution Engine and Morphological Stability Manager define the geometric deformation of a granular boundary due to spatially varying sediment transport and mathematically govern the temporal synchronization between the fast-moving fluid continuum and the slow-moving morphological continuum to maintain numerical stability.

Fundamental terminology:

Bed elevation (zb): The vertical distance from a fixed datum to the fluid-solid interface.

Porosity (λ): The volume fraction of voids (water or air) within the bulk sediment matrix.

Morphological wave celerity (cm): The propagation speed of a bed perturbation (e.g., a migrating sand dune).

Courant–Friedrichs–Lewy (CFL) condition: A necessary mathematical condition for the convergence of explicit finite-difference schemes, dictating that the numerical domain of dependence must encompass the physical domain of dependence.

Morphological Acceleration Factor (Mf): A scaling multiplier used to decouple and accelerate the morphological time scale relative to the hydrodynamic time scale.

Governing formulae:

1D Exner Equation (Conservation of Bed Material):

1-λ∂zb∂t+∂qs∂x=0

1-λ∂zb∂t+∂qs∂x=0

Where: zb= bed elevation (m) t= time (s) x= longitudinal spatial coordinate (m) λ= bed porosity (dimensionless) qs= volumetric sediment transport rate per unit channel width (m2/s)

Accelerated Morphological Equation:

1-λ∂zb∂thydro+Mf∂qs∂x=0

1-λ∂zb∂thydro+Mf∂qs∂x=0

Where: thydro= hydrodynamic time scale (s) Mf=ΔtmorphΔthydro= Morphological Acceleration Factor (dimensionless)

Morphological Wave Celerity:

cm=11-λ∂qs∂zb

cm=11-λ∂qs∂zb

Where: cm= celerity of the bed disturbance (m/s)

Global CFL Stability Condition (Time-Stepper):

Δt≤minΔx∣U∣+ghΔx∣cm∣

Δt≤minΔx|U|+gh,Δx|cm|

Where: Δt= numerical time step (s) Δx= spatial grid resolution (m) U= cross-sectional average fluid velocity (m/s) h= flow depth (m) g= gravitational acceleration (m/s2)

Assumptions and validity:

The Exner equation assumes that suspended load is negligible or in perfect equilibrium, meaning bed deformation is driven entirely by bedload divergence.

The bed porosity (λ) is assumed constant over depth and time.

The use of Mfassumes that hydrodynamic transients (like flood wave propagation) dissipate much faster than the bed geometry changes, allowing quasi-steady hydrodynamic states over scaled morphological intervals.

Boundary of applicability: The 1D Exner equation fails in regions of severe lateral channel migration (meandering) or bank collapse. High values of Mfwill cause unphysical numerical artifacts if the boundary conditions (e.g., inflow hydrograph) change rapidly relative to the bed response.

Mathematical classification: The Exner equation is a non-linear, hyperbolic partial differential equation (PDE). The CFL condition is a numerical inequality constraint.

============================================================

Derivation 1: The Exner Equation

Starting principle: Conservation of mass for the solid granular phase.

Consider an Eulerian control volume of length Δxand unit width (B=1). The active bed layer has a vertical thickness defined by the bed elevation zbabove an arbitrary impermeable datum z0=0.

The total macroscopic volume of the bed element is V=Δx⋅zb.

The volume of solid particles Vswithin this element depends on the void fraction (porosity, λ):

Vs=1-λΔxzb

The time rate of change of the solid volume within the control volume is:

∂Vs∂t=1-λΔx∂zb∂t

The net volumetric flux of sediment entering and leaving the control boundaries at xand x+Δxis:

Δqs=qsx-qsx+Δx

Using a first-order Taylor series expansion for the spatial variation of flux:

qsx+Δx=qsx+∂qs∂xΔx

Substituting this expansion into the net flux expression:

Δqs=qsx-qsx+∂qs∂xΔx=-∂qs∂xΔx

Δqs=qsx-qsx+∂qs∂xΔx=-∂qs∂xΔx

Equating the rate of change of solid volume to the net flux (Mass in - Mass out):

1-λΔx∂zb∂t=-∂qs∂xΔx

1-λΔx∂zb∂t=-∂qs∂xΔx

Dividing by the spatial increment Δxyields the continuous differential equation:

1-λ∂zb∂t+∂qs∂x=0

1-λ∂zb∂t+∂qs∂x=0

Derivation 2: Morphological Celerity and Stability

Starting principle: Chain rule of differentiation to express flux divergence in terms of bed elevation.

Assume the sediment flux qsis primarily a function of the local flow velocity, which is strongly coupled to the bed elevation zbunder constant discharge Q.

Rewrite the spatial divergence of flux as:

∂qs∂x=∂qs∂zb∂zb∂x

∂qs∂x=∂qs∂zb∂zb∂x

Substitute this into the Exner equation:

1-λ∂zb∂t+∂qs∂zb∂zb∂x=0

1-λ∂zb∂t+∂qs∂zb∂zb∂x=0

Divide by 1-λ:

∂zb∂t+11-λ∂qs∂zb∂zb∂x=0

∂zb∂t+11-λ∂qs∂zb∂zb∂x=0

This formulation takes the form of the kinematic wave equation ∂ϕ∂t+c∂ϕ∂x=0. By analogy, the wave celerity cmis:

cm=11-λ∂qs∂zb

cm=11-λ∂qs∂zb

B. PHYSICAL INTERPRETATION The mathematical structure of ∂qs∂xdictates morphological response. If flow accelerates over a bump, shear stress increases, causing more sediment to leave the downstream face of the control volume than entered the upstream face (∂qs∂x>0). To balance the equation, ∂zb∂tmust be negative, meaning the bed degrades (scours). Conversely, if flow decelerates, flux converges (∂qs∂x<0), ∂zb∂tbecomes positive, and the bed aggrades (deposits).

The celerity cmproves that bed forms (like dunes or anti-dunes) propagate mathematically as waves. Because qsis small compared to water discharge, cmis typically orders of magnitude smaller than fluid wave celerity.

============================================================

MATHEMATICAL REPRESENTATION

The 1D Exner equation is a specific spatial reduction of a general vectorial conservation law. In a full 2D or 3D continuum representation, the sediment transport rate is not a scalar qs, but a vector field qsdefined on the x-yhorizontal plane.

qs=qsxe1+qsye2

qs=qsxe1+qsye2

Where e1and e2are orthogonal Cartesian basis vectors.

B. MECHANICAL FORMULATION In a fully coupled 3D continuum, the riverbed is defined as an implicit kinematic surface function Fx,y,z,t=0:

Fxyzt=z-zbxyt=0

Fx,y,z,t=z-zbx,y,t=0

The normal vector to this topological surface is given by the spatial gradient ∇F.

For the bed to evolve without interstitial voids spontaneously appearing or disappearing, the material derivative (the substantial rate of change) of this surface tracking function must balance the volumetric deposition or entrainment rate (denoted by a scalar source term E-D, Entrainment minus Deposition).

C. DERIVATION AND VERIFICATION Applying the divergence theorem to a 3D control volume intersecting the bed boundary, the local rate of change of the boundary elevation is driven by the 2D divergence operator applied to the flux vector field.

The vectorial Exner equation takes the form:

1-λ∂zb∂t+∇⋅qs=0

1-λ∂zb∂t+∇⋅qs=0

Expanding the divergence operator in Cartesian coordinates:

1-λ∂zb∂t+∂qsx∂x+∂qsy∂y=0

1-λ∂zb∂t+∂qsx∂x+∂qsy∂y=0

In this tensorial continuum view, the flux vector qsdoes not merely align with the depth-averaged velocity vector U. It is deflected by secondary circulation and transverse bed slopes (gravity pull on particles along the transverse gradient ∂zb/∂y).

The flux vector is defined constitutively as:

qs=∣qs∣cosδse1+sinδse2

qs=|qs|cosδse1+sinδse2

Where δsis the deviation angle of the solid transport from the fluid streamline due to transverse gravity effects.

This verifies that the 1D model used in Phase 1 assumes perfect symmetry in the y-direction (∂/∂y=0), mathematically restricting the flux vector to align perfectly with the longitudinal domain.

============================================================

CONTINUUM DESCRIPTION

The fully coupled morphodynamic system comprises two intersecting continua: a fast-responding hyperbolic fluid system governed by Saint-Venant, and a slow-responding hyperbolic granular system governed by Exner. The independent variables are xand t. The unknown state vector for the coupled system is expanded to Usys=h,U,zbT.

B. LOCAL GOVERNING EQUATIONS The coupled system can be expressed as a single vector conservation equation with a non-conservative source term:

∂∂thUhzb+∂∂xUhU2h+12gh2qs1-λ=0ghS0-Sf0

∂∂thUhzb+∂∂xUhU2h+12gh2qs1-λ=0ghS0-Sf0

To analyze the temporal behavior and stability of this system, we express it in quasi-linear Jacobian form:

∂Usys∂t+A∂Usys∂x=S

∂Usys∂t+A∂Usys∂x=S

Where Ais the 3×3Jacobian matrix containing the partial derivatives of the flux vectors with respect to the state variables.

C. CLOSURE AND PHYSICAL CONSISTENCY (The Stability Manager) The physical necessity of the Morphological Time-Stepper arises directly from the mathematical properties of the Jacobian matrix A. Solving for the eigenvalues (Λ) of Ayields three characteristic wave speeds:

Λ1≈U+gh(Downstream fluid gravity wave)

Λ2≈U-gh(Upstream fluid gravity wave)

Λ3≈11-λ∂qs∂zb(Morphological bed wave)

The Stiffness Problem: For typical Froude numbers in rivers, |Λ1|∼O100 to 101 m/s, while the morphological wave |Λ3|∼O10-4 to 10-5 m/s.

Because numerical stability (the CFL condition) strictly requires Δtto be constrained by the maximum eigenvalue (Λ1), the simulation is forced to take sub-second time steps. To simulate 10 years of bed evolution (zb), the solver would require billions of hydrodynamic calculations, overwhelming the CPU.

The Decoupling Solution: The Stability Manager resolves this mathematical stiffness by decoupling the continuum. It assumes the fluid system adjusts instantaneously to the slow bed changes.

The hydrodynamic engine integrates at Δthydro≈0.1 susing the fast eigenvalues.

The Exner engine multiplies the resulting flux gradient by the Morphological Acceleration Factor (Mf=100 to 1000).

The Exner equation updates the geometry zbat a simulated Δtmorph=Mf⋅Δthydro.

The flow solver updates the spatial field based on the new geometry, closing the feedback loop without mathematically violating the Courant stability limit of the shock-capturing scheme.

Morphological Coupling & Stability Manager (The Time-Stepper)

SECTION 1 — FUNDAMENTAL DEFINITIONS AND GOVERNING FORMULAE

Definition:

The Morphological Coupling and Stability Manager (the Time-Stepper) is the mathematical and numerical framework that governs the temporal integration of the coupled hydrodynamic and morphodynamic continuum. It resolves the severe characteristic stiffness inherent in the coupled system by managing the disparate propagation speeds of fluid waves and bed-deformation waves, ensuring that explicit numerical schemes converge without violating local causality.

Fundamental terminology:

Characteristic eigenvalue (Λ): The local propagation celerity of a physical disturbance (wave) along a characteristic curve in the space-time (x-t) domain.

System stiffness: A mathematical condition arising in a system of differential equations where the eigenvalues span several orders of magnitude, forcing numerical solvers to take excessively small-time steps.

Courant–Friedrichs–Lewy (CFL) condition: A necessary condition for the stability of explicit finite-difference schemes, requiring the numerical domain of dependence to encompass the analytical domain of dependence.

Courant number (Cr): A dimensionless number quantifying the ratio of the physical wave speed to the numerical grid speed.

Morphological Acceleration Factor (Mf): A temporal scaling parameter utilized to artificially accelerate bed evolution while maintaining quasi-steady hydrodynamics.

Governing formulae:

System Vector Equation:

∂U∂t+AU∂U∂x=SU

Where:

U = state vector of the continuum

AU=∂F∂U = Jacobian flux matrix

SU = source term vector

Courant–Friedrichs–Lewy (CFL) Stability Criterion:

Δt≤CmaxΔxmax|Λk|

Where:

Δt = numerical time step (s)

Δx = spatial grid size (m)

Λk = the k-th eigenvalue of the Jacobian matrix A (m/s)

Cmax = maximum permissible Courant number (Cmax≤1 for explicit schemes)

Morphological Time-Scaling:

Δtmorph=Mf⋅Δthydro

Where:

Δtmorph = time step applied to the Exner equation (s)

Δthydro = time step applied to the Saint-Venant equations (s)

Mf = Morphological Acceleration Factor (dimensionless)

Assumptions and validity:

The formulation assumes the system is hyperbolic, meaning all eigenvalues of A are real and distinct.

The use of Mf≫1 assumes that the hydrodynamic transient waves (e.g., flood routing) dampen or exit the physical domain much faster than the bed geometry undergoes macroscopic changes (quasi-steady flow assumption).

The formulation is invalid for highly unsteady, flash-flood morphodynamics where the rate of change of the hydrograph dQ/dt is on the same order of magnitude as the morphological response.

Boundary of applicability:

If Mf is set excessively high, the decoupled system violates the conservation of mass, as the boundary conditions will artificially inject flow over a bed that has "fast-forwarded" into an incompatible geometry, inducing numerical shocks.

Mathematical classification:

The CFL condition is a mathematical inequality restricting the temporal discretization. The decoupled time-stepping is an asymptotic perturbation scheme applied to a system of non-linear hyperbolic partial differential equations.

SECTION 2 — MATHEMATICAL DERIVATION AND PHYSICAL INTERPRETATION

FIRST-PRINCIPLES DERIVATION OF SYSTEM STIFFNESS

To establish the necessity of the Stability Manager, we must derive the characteristic wave speeds of the coupled system from first principles.

Starting principle: The 1D continuity, momentum, and Exner equations expressed in primitive variables. Let u be the cross-sectional velocity, h be the flow depth, and zb be the bed elevation.

The continuity equation for water:

∂h∂t+h∂u∂x+u∂h∂x=0

The momentum equation for water (neglecting friction and viscosity for characteristic analysis):

∂u∂t+u∂u∂x+g∂h∂x+g∂zb∂x=0

The Exner equation, assuming solid flux qs=Asub where As and b are empirical constants (yielding ∂qs∂x=bAsub-1∂u∂x):

∂zb∂t+11-λbAsub-1∂u∂x=0

Let ξ=bAsub-11-λ. The Exner equation becomes:

∂zb∂t+ξ∂u∂x=0

Formulate the system in quasi-linear vector form ∂U∂t+A∂U∂x=0:

∂∂thuzb+uh0gug0ξ0∂∂xhuzb=000

The characteristic wave speeds are the eigenvalues Λ of the Jacobian matrix A. Set the determinant of A-ΛI to zero:

detu-Λh0gu-Λg0ξ-Λ=0

Expanding the determinant:

u-Λu-Λ-Λ-gξ-hg-Λ-0=0

Simplifying yields the characteristic cubic equation:

Λ3-2uΛ2+u2-gh-gξΛ+ugξ=0

To decouple the roots analytically, recognize that sediment flux is extremely small compared to water flux (ξ≪h). By applying regular perturbation theory, the roots asymptotically separate into:

Λ1,2≈u±gh (Hydrodynamic waves)

Λ3≈ugξu2-gh=u1-Fr2ξh (Morphodynamic wave)

PHYSICAL INTERPRETATION

The derivation proves that a fully coupled riverine system generates three distinct waves. Λ1 and Λ2 represent the downstream and upstream propagation of surface water waves, driven by gravity g and depth h. Their speeds are typically 1 to 10 m/s.

Λ3 represents the migration celerity of the riverbed itself (e.g., a sand dune moving downstream). Because ξ (the derivative of sediment flux) is infinitesimally small compared to flow depth h, |Λ3| is typically 10-5 m/s.

This mathematical discrepancy creates extreme system stiffness. If we integrate the equations simultaneously, the CFL condition dictates that the global time step Δt must be severely restricted by the fastest wave (Λ1≈10 m/s) to prevent the explicit spatial gradients from overshooting the physical domain of dependence.

Consequently, running the system to observe a 10-meter migration of a sand dune (which takes weeks physically) would require millions of microsecond time steps dictated by the surface water waves. The Stability Manager explicitly resolves this by separating the execution loops, leveraging the mathematical proof that Λ3≪Λ1.

============================================================

SECTION 3 — TENSORIAL FORMULATION AND MECHANICAL INTERPRETATION

MATHEMATICAL REPRESENTATION

The Jacobian matrix Aij=∂Fi∂Uj acts as a second-order tensor mapping the spatial gradient vector ∂U∂x to the temporal rate vector ∂U∂t in the abstract state space of the system, rather than in physical Euclidean space.

Because the system is strictly hyperbolic, this state-space tensor A possesses a complete set of linearly independent right eigenvectors Rk and left eigenvectors Lk, allowing it to be diagonalized:

A=RΛL

Where Λ is a diagonal tensor containing the eigenvalues Λ1,Λ2,Λ3.

MECHANICAL INTERPRETATION IN STATE SPACE

When the governing equations are multiplied by the left eigenvector tensor L, the coupled PDEs transform into a system of decoupled ordinary differential equations along specific space-time trajectories (characteristic curves):

L∂U∂t+A∂U∂x=0⟹∂W∂t+Λ∂W∂x=0

Here, W represents the Riemann Invariants—abstract mechanical quantities that remain perfectly conserved as they travel along their respective characteristic paths.

DERIVATION AND VERIFICATION OF DECOUPLING

The existence of the diagonal tensor Λ verifies that the physical system is essentially a superposition of decoupled advection processes operating at vastly different speeds.

Because the non-diagonal elements of Λ are zero, a disturbance in the Riemann invariant corresponding to the bed (W3) does not instantaneously mechanically deform the Riemann invariants of the water surface (W1,W2) and vice versa, provided the timescale of observation is sufficiently narrowed.

The Morphological Time-Stepper leverages this exact state-space orthogonality. By freezing W1 and W2 (assuming the hydrodynamic field has reached a steady-state boundary value equilibrium), the tensor equation reduces to a single scalar advection equation for the bed:

∂W3∂tmorph+Λ3∂W3∂x=0

This mechanically validates the use of the Morphological Acceleration Factor (Mf), as the state-space tensor proves the bed evolution is mathematically invariant to the high-frequency transient oscillations of the fluid phase.

SECTION 4 — CONTINUUM-LEVEL FORMULATION AND GOVERNING EQUATIONS

CONTINUUM DESCRIPTION

To formally map the decoupled time-stepping into a mathematically rigorous continuum description, we define two continuous, independent temporal dimensions:

A fast hydrodynamic time scale: t∼OL/U

A slow morphodynamic time scale: τ∼OL/Λ3

Let ϵ be the ratio of the solid-to-fluid propagation speeds:

ϵ=Λ3U≪1

We define the slow morphological time coordinate as τ=ϵt.

LOCAL GOVERNING EQUATIONS via ASYMPTOTIC EXPANSION

Transform the temporal derivatives using the chain rule to incorporate both timescales:

∂∂ttotal=∂∂t+ϵ∂∂τ

Substitute this expansion into the hydrodynamic mass conservation equation:

∂h∂t+ϵ∂h∂τ+∂uh∂x=0

Because we are interested in the long-term evolution of the bed, we evaluate the system on the slow timescale τ. Over this scale, the high-frequency transient term ∂h∂t rapidly decays to zero as the fluid reaches boundary equilibrium.

As ϵ→0, the continuous hydrodynamic field equation simplifies to:

∂uh∂x=0⟹uh=Qx

The momentum equation similarly reduces to a steady-state ordinary differential equation for the backwater profile over the current bed topology.

The Exner equation, governed entirely by the slow material transport, is formulated strictly in the slow timescale τ:

1-λ∂zb∂τ+∂qs∂x=0

CLOSURE AND PHYSICAL CONSISTENCY

The governing equations now form a closed, mathematically consistent system that correctly represents the physics without suffering from numerical stiffness. The Time-Stepper executes this continuum theory through the following sequential numerical algorithm:

Hydrodynamic Stepping: Freeze τ. Integrate the Saint-Venant equations using Δthydro governed strictly by Δthydro≤Δx|U|+gh until the flow field U converges to a steady state ∂U∂t≈0 over the current bed geometry zb.

Flux Computation: Calculate the spatial distribution of boundary shear stress τb and the resulting volumetric sediment flux qs.

Morphological Stepping: Freeze the fluid continuum. Update the bed geometry by numerically integrating the Exner equation using a massive time step Δtmorph=Mf⋅Δthydro.

Boundary Update: Map the newly deformed topological boundary zbx,τ+Δτ back into the spatial domain of the hydrodynamic solver.

Iteration: Repeat the loop.

CONNECTION TO ENGINEERING APPLICATIONS

In the HydroMorph analytics suite, this explicit continuum formulation allows a basic i3 processor to simulate decades of riverbed shifting, reservoir sedimentation, or local bridge pier scouring in minutes. By mathematically proving via the Jacobian tensor and asymptotic expansion that the hydrodynamics can be treated as quasi-steady boundary value problems between morphodynamic jumps, the engine bypasses billions of unnecessary fluid-wave calculations, safely violating the morphological CFL condition without destroying the stability of the coupled continuum.

THE MICRO-SCALE BENTHIC-BOUNDARY FORMULATION

SECTION 1 — FUNDAMENTAL DEFINITIONS AND GOVERNING FORMULAE

- Definition:

The micro-scale benthic-boundary formulation describes the vertical distribution of suspended solid particles within a turbulent fluid shear flow. It quantifies the dynamic equilibrium between the downward gravitational settling of particles and the upward turbulent diffusion generated by boundary shear forces.

- Fundamental terminology:

- Shear velocity: A characteristic velocity scale representing the magnitude of the boundary shear stress acting on the fluid.

- Viscous sublayer: The infinitesimally thin fluid layer immediately adjacent to the solid boundary where turbulent fluctuations are damped, and viscous shear dominates.

- Settling velocity: The terminal vertical velocity of a solid particle falling through a quiescent fluid under the influence of gravity and fluid drag.

- Turbulent eddy diffusivity: A kinematic parameter characterizing the rate of mass transport via turbulent eddies in a fluid continuum.

- Rouse number: A dimensionless parameter expressing the ratio of particle settling velocity to the upward turbulent transport velocity.

- Governing formulae:

(a) Shear Velocity:

u*=τbρ

Where:

u* = shear velocity (m/s)

τb = basal boundary shear stress (N/m2)

ρ = fluid density (kg/m3)

(b) Particle Settling Velocity (Stokes' Law regime, spherical particles):

ws=ρs-ρgd218μ

Where:

ws = terminal settling velocity (m/s)

ρs = particle density (kg/m3)

g = gravitational acceleration (m/s2)

d = particle diameter (m)

μ = dynamic viscosity of the fluid (kg/(m⋅s))

(c) Turbulent Sediment Diffusivity (Parabolic Distribution):

εsz=κu*z1-zh

Where:

εsz = vertical turbulent sediment diffusivity at elevation z (m2/s)

κ = von Kármán constant (≈0.40, dimensionless)

z = vertical elevation above the bed (m)

h = total flow depth (m)

(d) Rouse Suspension Profile:

CzCa=h-zzah-aRo

Where:

Cz = volumetric sediment concentration at elevation z (dimensionless)

Ca = reference sediment concentration at reference elevation a (dimensionless)

a = reference elevation near the bed, typically defining the top of the bedload layer (m)

Ro = Rouse number (dimensionless), defined as:

Ro=wsκu*

- Assumptions and validity:

- The fluid is incompressible and the flow is steady, uniform, and fully rough turbulent.

- The sediment concentration is sufficiently dilute such that particle-particle interactions and turbulence damping by particles are negligible.

- The settling velocity ws is assumed constant and independent of the turbulence field.

- The turbulent momentum diffusivity (eddy viscosity) and sediment mass diffusivity are assumed to be identical (Reynolds analogy, turbulent Schmidt number Sct=1).

- Boundary of applicability:

The parabolic diffusivity profile and the resulting Rouse equation fail in the immediate vicinity of the free surface (z=h), where the predicted diffusivity drops to zero, which contradicts physical observations of finite surface mixing. It also fails within the viscous sublayer (z→0) where molecular diffusion supersedes turbulent diffusion.

- Mathematical classification:

The governing transport balance is a linear, second-order ordinary differential equation (ODE). The resulting Rouse profile is an exact algebraic solution to this differential boundary value problem.

============================================================

SECTION 2 — MATHEMATICAL DERIVATION AND PHYSICAL INTERPRETATION

A. FIRST-PRINCIPLES DERIVATION

Derivation 1: The Parabolic Eddy Diffusivity Distribution

Starting principle: The Boussinesq approximation for turbulent shear stress and the logarithmic velocity profile.

Assume a linear distribution of shear stress over the fluid depth h in steady, uniform open-channel flow:

τz=τb1-zh

Substitute the definition of shear velocity τb=ρu*2:

τz=ρu*21-zh

Introduce the Boussinesq hypothesis, which models Reynolds stress analogously to viscous stress using turbulent eddy viscosity νt:

τz=ρνtdudz

The law of the wall (logarithmic velocity profile) dictates the vertical velocity gradient:

dudz=u*κz

Substitute the velocity gradient into the Boussinesq equation:

τz=ρνtu*κz

Equate the linear shear stress distribution to the Boussinesq relation:

ρu*21-zh=ρνtu*κz

Solve algebraically for the eddy viscosity νt:

νtz=κu*z1-zh

Applying the Reynolds analogy, assume the turbulent diffusivity of suspended sediment εs equals the momentum eddy viscosity νt:

εsz=κu*z1-zh

Derivation 2: The Rouse Equation

- Starting principle: Conservation of scalar mass in 1D vertical steady state.

- The downward advective flux due to gravity is qdown=wsC.

- The upward diffusive flux due to turbulence is modeled by Fick's first law: qup=-εsdCdz.

At dynamic equilibrium, the net vertical flux is zero:

wsC+εsdCdz=0

Substitute the parabolic diffusivity εsz:

wsC+κu*z1-zhdCdz=0

Separate variables C and z:

dCC=-wsκu*z1-zhdz

Rewrite the spatial denominator using partial fractions. Note that 1z1-z/h=hzh-z:

hzh-z=1z+1h-z

Substitute the partial fraction expansion:

dCC=-wsκu*1z+1h-zdz

Integrate both sides from a reference elevation z=a (where C=Ca) to an arbitrary elevation z:

CaC1CdC=-wsκu*az1z+1h-zdz

Perform the definite integration:

lnC-lnCa=-wsκu*lnz-lnh-zaz

lnCCa=-wsκu*lnzh-z-lnah-a

Apply logarithmic properties:

lnCCa=wsκu*lnh-zzah-a

Exponentiate both sides, substituting Ro=ws/κu*:

CzCa=h-zzah-aRo

Dimensional verification: ws (m/s) and u* (m/s) render Ro dimensionless. The spatial term h-z/z is identical in units (m/m), rendering the final equation dimensionally homogeneous.

B. PHYSICAL INTERPRETATION

The advection-diffusion balance wsC=-εsdCdz physically represents a state where gravity continuously pulls particles downward, creating a concentration gradient (dCdz<0). This gradient induces an upward turbulent scattering of particles.

The Rouse number Ro dictates the shape of the profile. It is the ratio of the extraction mechanism (ws) to the suspension mechanism (κu*).

- If Ro≪1, turbulence dominates gravity. The exponent approaches zero, Cz/Ca→1, and the sediment is perfectly mixed uniformly over the depth (e.g., wash load).

- If Ro≈1, the terms are balanced, resulting in a pronounced curved profile where concentration decays logarithmically away from the bed.

- If Ro≫1, gravity heavily dominates. The concentration drops strictly to zero just above the reference height a, indicating that the particles cannot be suspended and must move exclusively as bedload.

SECTION 3 — TENSORIAL FORMULATION AND MECHANICAL INTERPRETATION

A. MATHEMATICAL REPRESENTATION

The scalar 1D advection-diffusion equation is a spatial reduction of the full 3D scalar transport equation acting within a turbulent continuum field.

We define a stationary Cartesian coordinate system xi, where x1 aligns with the flow, x2 is transverse, and x3 opposes gravity.

The relevant quantities are:

- The continuous scalar concentration field Cxi,t.

- The instantaneous fluid velocity vector field uixi,t.

- The settling velocity vector wsi, which is uniformly aligned with gravity: wsi=0,0,-ws.

- The total advective flux vector qi=Cui+wsi.

B. MECHANICAL FORMULATION

Applying Reynolds decomposition, the instantaneous scalar concentration and velocity fields are separated into temporal means (C, ui) and turbulent fluctuations (C', ui').

The ensemble-averaged transport equation for the scalar C involves the divergence of the mass flux:

∂C∂t+∂∂xiCui+wsi=-∂∂xiC'ui'+∂∂xiDm∂C∂xi

The term C'ui' is the turbulent mass flux vector. Mechanically, it represents the correlation between velocity fluctuations and concentration fluctuations.

To achieve closure, this turbulent flux vector is parameterized using a generalized gradient diffusion hypothesis via a second-order turbulent diffusivity tensor Dij:

C'ui'=-Dij∂C∂xj

Where Dij is mathematically analogous to the turbulent eddy viscosity tensor, dictating how scalar gradients in the xj direction produce turbulent fluxes in the xi direction.

C. DERIVATION AND VERIFICATION

- Assume molecular diffusion Dm is negligible compared to turbulent diffusion.

- Assume the flow is steady (∂/∂t=0) and fully developed in the horizontal plane (∂/∂x1=0, ∂/∂x2=0).

- The vertical mean fluid velocity u3 is zero in a steady uniform open-channel flow.

- The settling velocity vector simplifies to ws3=-ws.

- The spatial gradient vector simplifies to a single component ∂C/∂x3.

- The turbulent diffusivity tensor Dij reduces to its principal vertical component D33, which is scalar εs.

Substituting these reductions into the tensorial transport equation:

∂∂x3C-ws=-∂∂x3-D33∂C∂x3

Integrating once with respect to x3 (and noting that fluxes must be zero at the free surface):

-wsC=-D33∂C∂x3⟹wsC+εs∂C∂x3=0

This verifies that the 1D steady-state ODE utilized in Derivation 2 is the exact analytical reduction of the 3D tensorial scalar transport equation under the condition of strictly horizontal, fully developed turbulent flow.

The system is modeled as an interacting two-phase continuum (Eulerian-Eulerian framework), where the dilute solid granular phase (sediment) acts as a passive scalar dispersed within the primary fluid continuum (water). The spatial domain is a control volume bounded by the rigid, rough riverbed boundary at z=0 and the free surface at z=h. The independent variables are spatial elevation z and time t.

LOCAL GOVERNING EQUATIONS

The physical state of this interacting continuum is closed by defining equations for both momentum (to determine the turbulence field) and scalar mass (to determine the concentration field).

Fluid Momentum (RANS Equation, simplified for 1D vertical shear):

0=gSf+1ρ∂∂zρνt∂u∂z

This dictates the distribution of shear stress from the driving gravity gradient Sf to the boundary.

Dispersed Solid Mass (Scalar Transport Equation):

∂C∂t-ws∂C∂z=∂∂zεs∂C∂z

CLOSURE AND PHYSICAL CONSISTENCY

To close the continuous system, the relation between the fluid momentum field and the solid mass field must be established.

The turbulent diffusivity εs is linked to the momentum eddy viscosity νt via the turbulent Schmidt number Sct:

εs=νtSct

In the Rouse derivation, Sct is assumed to be exactly 1.0 (Reynolds analogy).

Boundary Conditions are strictly required to solve the differential system:

  - Kinematic Free Surface (z=h): The net flux of particles across the water surface must be zero.

εs∂C∂z+wsCz=h=0

  - Benthic Boundary (z=a): The system requires a Dirichlet boundary condition Ca=Ca immediately above the viscous sublayer. This is physically supplied by an empirical bed-entrainment function (such as the Garcia-Parker or van Rijn relations), which calculates Ca as a direct function of the basal shear stress τb.

CONNECTION TO ENGINEERING APPLICATIONS

In the execution of the Benthic-Boundary Engine (Phase 1 Micro-Scale), this continuum formulation is applied directly to quantify total suspended sediment load.

While the Meso-scale Exner engine calculates bedload transport based on τb, the Micro-scale engine utilizes the same τb to determine u*, which initializes the Rouse profile.

By numerically integrating the product of the logarithmic velocity profile uz and the Rouse concentration profile Cz from the reference height a to the surface h, the engine yields the suspended sediment transport capacity qss:

qss=ahuzCzdz

This provides the complete deterministic bridge between microscopic boundary layer kinematics (viscous sublayer shear) and macroscopic morphodynamic modeling (total sediment routing).

Phase 2 Developer Notes: Micro-Scale Benthic-Boundary Engine

To integrate the Benthic-Boundary mechanics into the HydroMorph suite as a stable, decoupled Streamlit/FastAPI module, we must break the continuous micro-scale physics into discrete computational blocks. While the Meso-scale (Exner) engine dealt with spatial gradients along the river (dx), this Micro-scale engine strictly solves vertical spatial gradients over the flow depth (dz).

Here is the architectural breakdown of the Benthic-Boundary Engine into four distinct sub-models:

  - Near-Bed Kinematics & Boundary Layer Engine

SECTION 1 — FUNDAMENTAL DEFINITIONS AND GOVERNING FORMULAE

- Definition: The Near-Bed Kinematics and Boundary Layer Engine mathematically defines the vertical distribution of horizontal fluid velocity above a rigid boundary. It models the transition of momentum transfer from purely molecular (viscous) mechanisms at the physical boundary to macroscopic turbulent (eddy) mechanisms in the fluid column, establishing the kinematic driving field for particle entrainment and advection.

- Fundamental terminology:

- Shear velocity: A characteristic kinematic scale representing the magnitude of the boundary shear stress expressed in units of velocity.

- Viscous sublayer: The infinitesimally thin fluid region immediately adjacent to a solid boundary where turbulent velocity fluctuations are completely damped, and momentum transfer is governed entirely by molecular viscosity.

- Roughness Reynolds number: A dimensionless parameter quantifying the ratio of the physical bed roughness height to the viscous sublayer thickness, used to classify the hydraulic regime of the bed.

- Law of the Wall: A universal logarithmic relationship describing the vertical velocity gradient in a fully developed turbulent boundary layer.

- Governing formulae:

(a) Shear Velocity:

u*=τbρ

u*=τbρ

Where: u*= shear velocity (m/s) τb= basal boundary shear stress (N/m2) ρ= fluid density (kg/m3)

(b) Viscous Sublayer Thickness:

δv=11.6νu*

δv=11.6νu*

Where: δv= theoretical thickness of the viscous sublayer (m) ν= kinematic viscosity of the fluid (m2/s) 11.6= empirical intersection constant (dimensionless)

(c) Roughness Reynolds Number:

Re*=u*ksν

Re*=u*ksν

Where: Re*= roughness Reynolds number (dimensionless) ks= Nikuradse equivalent sand grain roughness (m)

(d) Prandtl-von Kármán Law of the Wall (Logarithmic Velocity Profile):

uz=u*κlnzz0

uz=u*κlnzz0

Where: uz= time-averaged longitudinal velocity at elevation z(m/s) z= vertical elevation above the physical bed (m) κ= von Kármán constant (≈0.40, dimensionless) z0= theoretical hydrodynamic zero-velocity datum (m)

- Assumptions and validity:

- The fluid is incompressible and Newtonian.

- The flow is steady, uniform, and fully developed, implying the boundary layer occupies the entire flow depth.

- The shear stress in the near-bed region (the inner region) is assumed constant and equal to the boundary shear stress τb.

- The length scale of turbulent eddies increases linearly with distance from the boundary.

- Boundary of applicability: The logarithmic profile fails in the outer region of the flow (typically z>0.2h) where the velocity distribution is influenced by the free surface (requiring a wake function correction). It also fails inside the viscous sublayer (z<δv), where the velocity profile is linear, not logarithmic.

- Mathematical classification: The shear velocity is an algebraic definition. The Law of the Wall is an exact analytical solution to a simplified differential equation for momentum transport.

============================================================ SECTION 2 — MATHEMATICAL DERIVATION AND PHYSICAL INTERPRETATION

A. FIRST-PRINCIPLES DERIVATION

Derivation 1: The Law of the Wall

- Starting principle: Prandtl’s Mixing Length Theory for turbulent shear stress.

- Assume steady, uniform flow where the total shear stress τnear the boundary is approximately equal to the basal shear stress τb.

- The turbulent shear stress is modeled as the transfer of momentum by fluid parcels moving vertically over a characteristic mixing length l:

τb=ρl2dudz2

τb=ρl2dudz2

- Assume that the mixing length lis strictly restricted by the presence of the solid boundary and grows linearly with elevation z:

l=κz

l=κz

Where κis the proportionality constant (von Kármán constant).

- Substitute the mixing length into the shear stress equation:

τb=ρκz2dudz2

τb=ρκz2dudz2

- Divide by ρand substitute the definition of shear velocity u*2=τb/ρ:

u*2=κ2z2dudz2

u*2=κ2z2dudz2

- Take the square root of both sides to isolate the velocity gradient:

u*=κzdudz

u*=κzdudz

- Rearrange into a separable differential equation:

du=u*κdzz

du=u*κdzz

- Integrate both sides:

du=u*κ1zdz

du=u*κ1zdz

uz=u*κln⁡z+C

uz=u*κlnz+C

- Apply the kinematic boundary condition. Define z0as the theoretical elevation where the logarithmic velocity extrapolates to zero, such that uz0=0:

0=u*κln⁡z0+C⟹C=-u*κln⁡z0

0=u*κlnz0+C⟹C=-u*κlnz0

- Substitute the constant of integration back into the profile:

uz=u*κln⁡z-u*κln⁡z0=u*κlnzz0

uz=u*κlnz-u*κlnz0=u*κlnzz0

Derivation 2: The Viscous Sublayer Thickness

- Starting principle: Conservation of momentum exclusively within the sublayer.

- Very close to the wall, turbulent eddies are suppressed by the physical boundary (l→0). The shear stress is governed entirely by Newton’s Law of Viscosity:

τb=μdudz

τb=μdudz

- Since τbis constant near the wall, integrate with respect to z, applying the no-slip boundary condition u0=0:

uz=τbμz

uz=τbμz

- Divide by u*to express the relation in dimensionless inner variables. Let u+=u/u*and z+=zu*/ν(where ν=μ/ρ):

uu*=ρu*2μu*z=u*zν⟹u+=z+

uu*=ρu*2μu*z=u*zν⟹u+=z+

This proves the velocity profile is strictly linear in the viscous sublayer.

- In dimensionless variables, the empirically fitted logarithmic profile (from Derivation 1) in the turbulent region is:

u+=1κln⁡z++B

u+=1κlnz++B

For a hydraulically smooth wall, experiments yield κ=0.40and B=5.5.

- The theoretical boundary of the viscous sublayer δvis defined at the geometric intersection of the linear inner profile and the logarithmic outer profile. Equating the two profiles:

z+=10.4ln⁡z++5.5

z+=10.4lnz++5.5

- Solving this transcendental equation iteratively for z+yields z+≈11.6.

- Substituting the definition of z+back into physical variables:

δvu*ν=11.6⟹δv=11.6νu*

δvu*ν=11.6⟹δv=11.6νu*

Dimensional verification: δv=1⋅m2/s/m/s=m, ensuring dimensional homogeneity.

B. PHYSICAL INTERPRETATION

The shear velocity u*is not a physical velocity of the fluid; it is a mathematical construct representing the intensity of the boundary shear stress. It defines the characteristic speed at which momentum diffuses toward the boundary.

The viscous sublayer thickness δvis inversely proportional to u*. Physically, as the flow accelerates and boundary shear increases, the turbulent energy penetrates deeper toward the bed, aggressively crushing and thinning the viscous sublayer.

The interaction between the physical roughness elements (ks) and δvdictates the hydraulic regime via Re*:

- If Re*<5(ks≪δv), the roughness elements are completely submerged within the viscous sublayer. The fluid glides over a smooth shear plane, and z0∝ν/u*.

- If Re*>70(ks≫δv), the roughness elements physically puncture the viscous sublayer, generating form drag and shedding turbulent wakes directly into the fluid. The viscous sublayer is physically destroyed, and z0∝ks.

============================================================ SECTION 3 — TENSORIAL FORMULATION AND MECHANICAL INTERPRETATION

A. MATHEMATICAL REPRESENTATION

The velocity profile uzis the spatial distribution of the u1component of the velocity vector field uixj. The governing dynamics emerge from the total effective stress tensor τijtotal, which comprises a viscous tensor and a Reynolds stress tensor.

Let x1align with the mean flow direction and x3align with the wall-normal (vertical) direction.

B. MECHANICAL FORMULATION

The total deviatoric stress tensor for an incompressible turbulent fluid is defined as:

τijtotal=2μSij-ρui'uj'

τijtotal=2μSij-ρui'uj'

Where: Sij=12∂ui∂xj+∂uj∂xi= mean rate-of-strain tensor -ρui'uj'= Reynolds stress tensor (momentum flux due to velocity fluctuations)

For steady, uniform, 1D open-channel flow, the mean velocity vector simplifies to ui=u1x3,0,0. The only non-zero component of the mean rate-of-strain tensor related to shear is S13=S31=12∂u1∂x3.

Consequently, the only active component of the stress tensor transmitting momentum toward the boundary is τ13total:

τ13total=μ∂u1∂x3-ρu1'u3'

τ13total=μ∂u1∂x3-ρu1'u3'

C. DERIVATION AND VERIFICATION

The kinematics of the boundary layer engine mathematically govern the spatial partition of this stress tensor component.

- At the physical boundary (x3=0): Due to the no-slip condition, all velocity fluctuations vanish (u1'=0,u3'=0). The Reynolds stress tensor evaluates to strictly zero. The stress tensor collapses to purely molecular viscosity:

τ13totalx3=0=μ∂u1∂x3

τ13total​x3=0=μ∂u1∂x3

This mechanical reduction verifies Derivation 2, proving that within the region where x3<δv, the continuum behaves as a laminar Stokes flow, generating the linear velocity profile.

- In the inertial sublayer (x3≫δv): The molecular viscosity μis orders of magnitude smaller than the turbulent macroscopic momentum exchange. The viscous term becomes mathematically negligible:

τ13total≈-ρu1'u3'

τ13total≈-ρu1'u3'

By invoking the Boussinesq approximation, the Reynolds stress tensor is parameterized using an eddy viscosity scalar νt:

-ρu1'u3'=ρνt∂u1∂x3

-ρu1'u3'=ρνt∂u1∂x3

Setting νt=κu*x3recovers the Law of the Wall.

This verifies that the near-bed kinematics are not arbitrary empirical curves, but exact spatial evaluations of the total stress tensor transitioning from its viscous components to its turbulent components along the x3coordinate axis.

============================================================ SECTION 4 — CONTINUUM-LEVEL FORMULATION AND GOVERNING EQUATIONS

A. CONTINUUM DESCRIPTION

The near-bed region is idealized as a 1D vertical Eulerian fluid continuum bounded by a rigid plane at z=0. The independent variable is the spatial elevation z. The primary field variable is the mean horizontal velocity uz.

B. LOCAL GOVERNING EQUATIONS

The fluid continuum is governed by the Reynolds-Averaged Navier-Stokes (RANS) equations. For steady, fully developed, uniform flow in an open channel driven by gravity, the longitudinal momentum balance simplifies significantly.

- Starting from the general Cauchy momentum equation:

ρDuiDt=ρgi+∂σji∂xj

ρDuiDt=ρgi+∂σji∂xj

- Expanding into the x1direction, assuming steady (∂/∂t=0) and uniform (∂/∂x1=0) flow, the convective acceleration terms vanish. The driving gravity term is gsinθ≈gS0.

- The local differential equation for momentum conservation becomes:

0=ρgS0+∂τ13total∂x3

0=ρgS0+∂τ13total∂x3

- Integrating with respect to x3(from the free surface hwhere shear is zero, down to elevation z):

τ13totalz=ρgS0h-z

τ13totalz=ρgS0h-z

- Very close to the bed (z≪h), the term h-z≈h. Therefore, the momentum equation dictates a region of constant stress:

τ13total≈ρghS0=τb

τ13total≈ρghS0=τb

C. CLOSURE AND PHYSICAL CONSISTENCY

The RANS momentum equation dictates that the shear stress is constant near the bed, but it cannot solve for the velocity field uzwithout a constitutive closure relationship.

The system is closed by substituting the turbulent parameterization τ13=ρκu*zdudzinto the constant-stress approximation derived from the RANS momentum equation:

ρu*2=ρκu*zdudz

ρu*2=ρκu*zdudz

This forms a closed, exact first-order ordinary differential equation that directly yields the logarithmic Law of the Wall upon integration. The physical consistency of the system is maintained by the boundary conditions: uz0=0provides the necessary integration constant, while ksdefines the topological scale of z0.

D. CONNECTION TO ENGINEERING APPLICATIONS

In the context of the Phase 2 Micro-Scale Benthic-Boundary Engine of the HydroMorph suite, this continuum formulation provides the foundational hydrodynamic array necessary to solve the macroscopic sediment transport problem.

The generated uzvelocity array is strictly required to evaluate the horizontal flux of suspended sediment. Once the Rouse profile Czis computed, the continuous integration of the product of these two fields yields the total suspended sediment transport rate qss:

qss=z0huzCzdz

qss=z0huzCzdz

Without the mathematically rigorous generation of the uzprofile from the law of the wall, the sediment concentration would lack the advective velocity field required to transport mass through the computational domain, rendering the morphodynamic engine fundamentally incomplete.

2. Particle Settling & Entrainment Module (The Boundary Condition Solver)

Before calculating how particles mix, the engine must determine how fast they fall and how many are ripped from the bed. This module establishes the critical Dirichlet boundary condition required to solve the differential equations later.

- Core Physics: Fluid Drag, Terminal Settling Velocity (ws), and Near-bed Reference Concentration (Ca).

- Mechanism:

  - Computes the settling velocity (ws) for the given particle diameter (d) using Stokes' Law (for fine silts) or the Dietrich/Soulsby empirical curves (for sands).

  - Defines a reference elevation (a), typically taken as the top of the bedload layer or proportional to dune height.

  - Computes the reference concentration (Ca) at elevation z=a using a deterministic entrainment function (e.g., Garcia-Parker or van Rijn), which correlates near-bed turbulence to particle lift.

- Outputs: Settling velocity (ws) and reference concentration scalar (Ca).

3. Turbulent Eddy Diffusivity Solver

This module maps the turbulent scattering power of the fluid. It defines how strongly the Reynolds stresses diffuse scalar quantities (like sediment) at different depths.

- Core Physics: Boussinesq Approximation and the Reynolds Analogy.

- Mechanism: Using the shear velocity (u*) and the von Kármán constant (κ≈0.40), it constructs the parabolic eddy diffusivity profile over the entire depth (h). The mixing is zero at the bed and free surface, and maximum at mid-depth.

εsz=κu*z1-zh

- Outputs: The vertical turbulent diffusivity array (εsz).

4. Suspension Profiler & Integration Engine (The Rouse Solver)

This is the core physics solver of the UI. It balances the downward gravitational settling against the upward turbulent diffusion to compute the final particle distribution and total suspended mass.

- Core Physics: 1D Advection-Diffusion ODE and the Rouse Number (Ro).

- Mechanism:

  - Calculates the dimensionless Rouse number (Ro=ws/κu*).

  - Solves the scalar transport ODE to generate the vertical concentration profile array Cz using the boundary condition Ca from Sub-model 2.

  - Integrates the product of the concentration array and the velocity array over the vertical depth to compute the total suspended sediment flux (qss):

qss=ahuzCzdz

- Outputs: The Rouse concentration array Cz (for Streamlit/Plotly 2D plotting), the Rouse Number (Ro), and the macroscopic suspended sediment transport capacity (qss).

Streamlit Integration Notes

For your Python architecture, Sub-models 1, 2, and 3 are instantaneous, computationally cheap scalar/array operations. Sub-model 4 involves numerical integration (scipy.integrate.simps or numpy.trapz). Because this entire engine avoids heavy time-stepping (it operates at a steady-state vertical snapshot), this entire 4-part module can run synchronously inside Streamlit without needing the FastAPI background decoupling that the Exner engine requires.

For the Particle Settling & Entrainment Module (Sub-model 2), break down the Garcia-Parker entrainment function. How do we calculate the reference concentration Ca mathematically?

FUNDAMENTAL DEFINITIONS AND GOVERNING FORMULAE

- Definition:

The Garcia-Parker entrainment function is an empirical, physically-scaled constitutive relation that quantifies the dimensionless volumetric rate at which bed sediment is lifted into suspension by near-bed turbulent bursts. It serves as the mathematical closure to determine the near-bed reference concentration Ca required to solve the suspended sediment transport field.

- Fundamental terminology:

- Explicit particle Reynolds number (Rp): A dimensionless parameter quantifying the ratio of inertial to viscous forces acting on a sediment particle, independent of the flow field.

- Skin friction shear velocity (u*'): The component of shear velocity generated purely by grain roughness, excluding the macroscopic form drag caused by bedforms (like dunes).

- Entrainment rate (Es): The dimensionless volumetric flux of sediment lifted from the bed into the fluid continuum per unit area.

- Similarity variable (Zu): A dimensionless scaling group that correlates the turbulent lifting forces to gravitational settling, modified by the particle Reynolds number.

- Governing formulae:

(a) Explicit Particle Reynolds Number:

Rp=Rggd50d50ν

Where:

Rp = explicit particle Reynolds number (dimensionless)

Rg=ρs-ρρ = submerged specific gravity of sediment (dimensionless, typically ≈1.65 for quartz)

g = gravitational acceleration (m/s2)

d50 = median particle diameter (m)

ν = kinematic viscosity of water (m2/s)

(b) Garcia-Parker Similarity Variable:

Zu=u*'wsRp0.6

Where:

Zu = similarity variable (dimensionless)

u*' = skin friction shear velocity (m/s)

ws = terminal settling velocity (m/s)

(c) Garcia-Parker Entrainment Function:

Es=AZu51+A0.3Zu5

Where:

Es = dimensionless volumetric entrainment rate (dimensionless)

A = empirical coefficient (A=1.3×10-7 for uniform sediment)

0.3 = maximum theoretical volumetric concentration limit at the bed

(d) Equilibrium Reference Concentration:

Ca=Es

Where:

Ca = volumetric sediment concentration at the reference elevation z=a (dimensionless)

- Assumptions and validity:

- The formulation assumes the flow has reached a state of dynamic equilibrium, where the upward turbulent flux of sediment perfectly balances the downward gravitational settling flux.

- The reference elevation a is strictly defined in the Garcia-Parker formulation as a=0.05h (5% of the total flow depth).

- The empirical coefficients are fitted for non-cohesive, uniform sands and silts. It requires modifications (hiding functions) for widely graded sediment mixtures.

- Boundary of applicability:

The function neglects entrainment driven by macroscopic flow separation (wake turbulence) behind bedforms, as it strictly utilizes the skin friction shear velocity u*'. If Zu is extremely small (flow is too weak), Es approaches zero, indicating all transport occurs strictly as bedload.

- Mathematical classification:

The similarity variable and entrainment functions are non-linear empirical algebraic relations. The equilibrium statement Ca=Es is an algebraic mass balance.

============================================================

SECTION 2 — MATHEMATICAL DERIVATION AND PHYSICAL INTERPRETATION

A. FIRST-PRINCIPLES DERIVATION OF THE EQUILIBRIUM BOUNDARY CONDITION

While the formula for Es is empirically derived from experimental data, the equivalence Ca=Es must be derived from the conservation of mass at the bed interface.

- Starting principle: Local conservation of sediment mass at the reference boundary z=a.

- Assume steady-state conditions (∂C/∂t=0).

- Define the upward volumetric sediment flux per unit area (qup) driven by turbulent bursts. Garcia and Parker define the dimensionless entrainment rate Es such that the upward flux is scaled by the settling velocity:

qup=Esws

- Define the downward volumetric sediment flux per unit area (qdown) driven by gravity. This is simply the local concentration multiplied by the settling velocity:

qdown=Caws

- At dynamic equilibrium, the net vertical mass flux across the boundary must be zero:

qup-qdown=0

- Substitute the flux expressions:

Esws-Caws=0

- Divide by ws (since ws≠0 for solid particles):

Ca=Es

This proves that the dimensionless entrainment rate Es mathematically equates to the reference volumetric concentration Ca.

B. PHYSICAL INTERPRETATION

The structure of the Garcia-Parker equation encodes critical physical mechanisms:

- The Zu5 dependence: The entrainment rate is governed by Zu raised to the 5th power. This demonstrates a violently non-linear sensitivity to fluid momentum. A mere 15% increase in shear velocity (u*') nearly doubles the sediment entrainment.

- The Rp0.6 scaling: The similarity variable Zu is not just the ratio of lift (u*') to fall (ws). The Rp0.6 term physically corrects for the viscous sublayer. Smaller particles have lower Rp and may be entirely submerged within the laminar sublayer, shielding them from the turbulent bursts that cause suspension.

- The asymptotic limit (Denominator): As Zu→∞ (e.g., during an extreme flood), Es does not increase infinitely.

limZu→∞AZu51+A0.3Zu5=0.3

This physically represents the maximum packing fraction of suspended grains. A volumetric concentration of 30% (Ca=0.30) represents the physical limit where particle-particle collisions completely damp turbulence and induce hindered settling, preventing further entrainment

The scalar entrainment flux qup is the 1D spatial reduction of the wall-normal component of the turbulent mass flux vector, which arises from the ensemble averaging of the scalar transport equation.

Let x3=z represent the coordinate axis normal to the riverbed. The turbulent mass flux vector is driven by the cross-correlation between concentration fluctuations C' and velocity fluctuations ui'.

B. MECHANICAL FORMULATION

In a 3D turbulent continuum, the Reynolds-averaged mass transport equation yields a turbulent mass flux term:

qt=C'ui'

At the bed boundary surface (x3=a), the physical mechanism of entrainment is represented strictly by the positive, wall-normal turbulent bursts (u3'>0). Therefore, the upward entrainment flux defined by Garcia and Parker is physically equivalent to the boundary evaluation of the third component of this cross-correlation vector:

qup=C'u3'​x3=a

C. DERIVATION AND VERIFICATION

- Equate the continuum turbulent flux to the empirical Garcia-Parker flux:

C'u3'​x3=a=Esws

- Substitute the empirical relation for Es:

C'u3'​x3=a=wsAZu51+A0.3Zu5

- This verifies that the Garcia-Parker equation operates mathematically as a constitutive closure relation for the wall-normal Reynolds mass flux at the continuum boundary.

- The extraction of u*' (skin friction) from the total boundary stress tensor τijtotal ensures that only the local viscous/turbulent shear stress acting directly on the grain surface—rather than the pressure drag acting on macro-bedforms—is used to compute Zu.

In the Benthic-Boundary Engine of the HydroMorph suite, the solid phase is modeled as a dilute continuous scalar field Cz dispersed within the fluid momentum field. The computational domain is bounded at the bottom by z=a.

B. LOCAL GOVERNING EQUATIONS

The steady-state spatial distribution of the sediment continuum is governed by the 1D advection-diffusion differential equation:

wsC+εsdCdz=0

Because this is a first-order ordinary differential equation with respect to z, it strictly requires one boundary condition to yield a unique solution.

C. CLOSURE AND PHYSICAL CONSISTENCY

Without the Particle Settling & Entrainment Module, the Rouse profile (derived in Section 2 previously) only provides the relative concentration Cz/Ca. It cannot quantify the actual mass of sediment in the water column.

The Garcia-Parker formulation mathematically closes the system by converting the hydrodynamic output (u*') into a rigid Dirichlet boundary condition:

Ca=AZu51+A0.3Zu5

Once Ca is numerically evaluated by this module, the Suspension Profiler (Sub-model 4) can solve the initial value problem.

D. CONNECTION TO ENGINEERING APPLICATIONS

In the execution of Phase 2 of your HydroMorph project, this sub-model acts as the critical bridge between flow intensity and absolute sediment mass.

By calculating Ca via Garcia-Parker and substituting it into the Rouse profile, the engine yields the absolute spatial field Cz in m3/m3. This allows the final integration:

qss=ahuzCzdz

To output qss in actual engineering units (m2/s or kg/s/m), proving the determinism of the suite: the water velocity explicitly dictates the boundary shear, the boundary shear explicitly dictates the Garcia-Parker entrainment mass, and the mass integrates into total macroscopic suspended transport.